SECTION VIII .

BILL OF QUANTITIES

|  | FOR COMMUNICATION BUILDING |  |  |  |
| --- | --- | --- | --- | --- |
| Item | Description | Unit | Qty |  |
|  | Sub-Structure |  |  |  |
| a | Excavate in top soil for an average depth of 150mm. The excavated soil shall be stockpiled for re-use in landscaping or for re-use as directed by the Engineer | m3 | 3.5 |  |
| b | Excavate to reduced levels for a depth not exceeding 1.2m from 150mm below the existing ground level for hardcore fill | m3 | 6.5 |  |
| c | Excavate in trenches 600mm wide for a depth not exceeding 1.5m for the strip footing | m3 | 21.0 |  |
| d | Dispose surplus excavated material as directed by the Engineer (provisional) | m3 | 10.0 |  |
| e | 50mm thick 1:3:6 mass concrete blinding to strip footings poured on or against earth. | m3 | 1.0 |  |
| f | Vibrated 1:2:4 mass concrete (maximum aggregate size 20mm) to 600 x 200 strip footing. | m3 | 3.0 |  |
| g | 200mm thick medium dressed natural stone walling bedded and jointed in 1:4 cement mortar in foundation walls | m2 | 34.5 |  |
| h | Hardcore fill to make up levels compacted in 150mm thick layers. Hardcore will be obtained off the site. | m3 | 5.5 |  |
| i | 50mm thick compacted murram on the hardcore filling | m2 | 22.0 |  |
| j | Termicide treatment ;Gladiator TC' or other approved insecticide treatment to blinded surfaces to be carried out by an approved specialist | m2 | 22.0 |  |
| k | Filling to excavations with selected excavated material compacted in 150mm thick layers | m3 | 10.5 |  |
| l | 1000 gauge damp proof membrane under the ground slab. Allow for 300mm laps | m2 | 22.0 |  |
| m | 150mm high timber formwork to the side of the ground slab | m | 9.0 |  |
| n | BRC A142 mesh reinforcement to B.S. 4483 in ground slab to allow for 300mm laps | m2 | 22.0 |  |
| o | Vibrated 1:2:4 reinforced concrete in 150mm thick ground slab | m3 | 3.5 |  |
|  | Super-Structure |  |  |  |
| p | 200mm wide Hessian based bituminous felt DPC to B.S. 747 bedded in 1:4 cement sand mortar | m | 23.0 |  |
| q | 200mm thick smooth dressed natural stone walling bedded and jointed in 1:4 cement mortar and pointing with neat recessed joints in super-structure walls | m2 | 49.0 |  |
| r | Sawn formwork for sides and soffits of beams not exceeding 200mm thick | m2 | 46 |  |
|  | Total carried to summary page |  |  |  |
| a | 200 x 200 x 200mm louvers vent blocks wall, bedded and jointed in 1:4 cement sand mortar; rectangular; pointed with neat recessed joints | nr | 18 |  |
| b | Vibrated 1:2:4 reinforced concrete in 200 x 450mm deep ring beam | m3 | 2.5 |  |
|  | High tensile steel to B.S. 4461 |  |  |  |
| c | Y10 in strip footing | kg | 30.0 |  |
| d | Y8 in strip footing | kg | 31.0 |  |
| e | Y12 in beams | kg | 35.0 |  |
| f | Y10 in beams | kg | 24.0 |  |
| g | Y8 in beams | kg | 33.0 |  |
| h | Y10 in roof slab | kg | 151.0 |  |
|  | Roof Structure |  |  |  |
| i | Vibrated 1:2:4 reinforced concrete in 150mm thick RC roof slab | m3 | 3.5 |  |
|  | Pitched roof members in sawn soft wood treated timber in trusses |  |  |  |
| j | 75 x 50mm rafters | m | 27.5 |  |
| k | 75 x 50mm bottom tie | m | 18.0 |  |
| l | 75 x 50mm struts and ties | m | 20.0 |  |
| m | 75 x 50mm purlins | m | 42.0 |  |
| n | 100 x 50mm wall plate | m | 19.5 |  |
| o | 200 x 25mm fascia board | m | 25.0 |  |
| p | 26gauge pre-painted IT4 sheets fixed in accordance with manufacturer's specifications | m2 | 36.5 |  |
| q | 300mm girth ridge to match the roof covering | m | 7.0 |  |
| r | Apply one coat of oil based under coat paint and two coats of oil based gloss paint to 200 x 25mm fascia board. | m | 25.0 |  |
|  | Doors |  |  |  |
| s | Supply and fix 2100mm high x 1000mm steel door complete with lock and handle as per drg. no SK 062449. This shall include painting with two coats of red oxide primer or equivalent two coats of gloss oil paint to the Client's colour. | nr. | 2.0 |  |
|  | Floor and wall Finishes |  |  |  |
| t | 25mm thick 1:3 cement sand screed steel trowelled smooth | m2 | 18.5 |  |
|  | Total carried to summary page |  |  |  |
|  | Floor and wall Finishes |  |  |  |
| a | 100m high x 20mm thick 1:3 cement sand skirting with a rounded top and curved junction | m | 18.0 |  |
| b | 20mm thick 1:3 cement sand plaster to walls | m2 | 64.0 |  |
| c | Apply two coats of oil based under coat and two coats of oil paint to the walls to the Client's colour | m2 | 64.0 |  |
| d | Horizontal key to the external face of walls | m2 | 49.0 |  |
|  | Roof Finishes |  |  |  |
| e | 20mm thick 1:3 cement sand plaster to the soffit of the roof slab | m2 | 18.5 |  |
|  | Rain Water Goods |  |  |  |
| f | 160mm diameter upvc half round plain rain water gutter to include hangers | m | 14.0 |  |
| g | 160mm diameter upvc half round gutter end-brown | nr | 2.0 |  |
| h | 160mm diameter upvc half round gutter end with a 110mm diameter sprout-brown | nr | 2.0 |  |
| i | 110mm diameter x 90o upvc brown waste water bend | nr | 2.0 |  |
| j | 110mm diameter x 45o upvc rain brown waste water bend | nr | 2.0 |  |
| k | 110mm diameter upvc brown waste water pipe | m | 12.0 |  |
| l | 5000 litre plastic water tank complete with an outlet pipe and tap. Allow for provision of 11/2'' overflow pipe | nr | 1.0 |  |
|  | Water Tank Plinth |  |  |  |
| m | Excavate in trench 600mm wide for a depth not exceeding 1.5m | m3 | 6.0 |  |
| n | 50mm thick 1:3:6 mass concrete blinding to strip footings poured on or against earth. | m3 | 0.3 |  |
| o | Vibrated 1:2:4 mass concrete (maximum aggregate size 20mm) to 600 x 200 strip footing. | m3 | 1.0 |  |
| p | Excavate in bulk in normal for the foundation of the water tank plinth for a depth not exceeding 1.2m | m3 | 4.0 |  |
| q | 200mm thick medium dressed natural stone walling bedded and jointed in 1:4 cement mortar in foundation walls | m2 | 10.0 |  |
| r | Hardcore fill to make up levels compacted in 150mm thick layers. Hardcore will be obtained off the site. | m3 | 4.0 |  |
| s | 50mm thick compacted murram on the hardcore filling | m2 | 13.0 |  |
|  | Total carried to summary page |  |  |  |
| a | Termicide treatment ;Gladiator TC' or other approved insecticide treatment to blinded surfaces to be carried out by an approved specialist | m2 | 13.0 |  |
| b | 1000 gauge damp proof membrane under the ground slab. Allow for 300mm laps | m2 | 13.0 |  |
| c | BRC A142 mesh reinforcement to B.S. 4483 in ground slab to allow for 300mm laps | m2 | 13.0 |  |
| d | Vibrated 1:2:4 reinforced concrete in 100mm thick ground slab. Allow for wood float finish while still wet | m3 | 1.5 |  |
| e | 150mm high timber formwork to the side of the ground slab | m | 6.5 |  |
|  | Electrical Installation |  |  |  |
| f | Allow a provisional sum for electrical installation as follows: |  |  |  |
|  | (i) Complete wiring 6 no. twin metallic 13-A socket outlet along the indoor cable tray |  |  |  |
|  | (ii) Complete wiring 3 no. 13 A socket outlet on wall |  |  |  |
|  | (iii) Complete wiring 1 no. 6-way consumer unit with 6x20A MCB. |  |  |  |
|  | (iv) 3 No. 4 feet fluorescent lamps complete (switch included) |  |  |  |
|  | (v) Complete wiring 2 no. security lights (front and back side of the repeater room (switch included) |  |  |  |
|  | NOTES |  |  |  |
|  | The electrical installation should be inclusive of supply and wiring (complete installation), inclusive of all appropriate cables. This includes the meter board |  |  |  |
|  | Please include a meter board all supply cables to the building to come from there. |  |  |  |
|  | Total carried to summary page |  |  |  |

NOTE:

Attached is a sample of the plan of a communication room page 121